| Engineering Document Number |  |
| --- | --- |
| Revision | V0.1 |
| Originator | Shurong huang |
| Date | 2015-03-25 |
| Project Code if Applicable | 429F |
| Department | Software group |
| Template Number |  |

Firmware Requirement Specifications for 429F Module

Approvers List

| Department/Title | Name | Signature | Date |
| --- | --- | --- | --- |

Revision History

| Revision | Date | Author | Comments |
| --- | --- | --- | --- |
| 0.1 | 2015-03-11 | Shurong huang |  |
| 0.2 | 2015-4-8 | Shuronghuang | Sch changed |
| 0.3 | 2015-8-13 | shuronghuang | SW instruction changed |
| 0.4 | 2017-5-25 | Shuronghuang | 增加PD解释 |
| 0.5 | 2018-8-9 | shuronghuang | 1x64换成memsswitch |
| 0.6 | 2019-2-27 | Shuronghuang | SW instruction modified |
| 0.8 | 2019-7-22 | shuronghuang | 增加对mcs18透传指令 |

Table of Contents

1.	Introduction	6

1.1.	Identification	6

1.2.	Purpose	6

1.3.	Document Conventions	6

1.4.	Intended Audience and Reading Suggestions	6

1.5.	Project Scope	6

1.6.	Reference	6

2.	Overall Description	7

2.1.	Overview Description	7

2.2.	Product Features	7

2.3.	User Classes and Characteristics	7

2.4.	Operating Environment	7

2.4.1.	Hardware Platform	7

2.4.2.	Operating System	7

2.4.3.	Other Software Components or Applications	7

2.5.	Design and Implementation Constraints	7

2.6.	Assumptions and Dependencies	7

3.	External Interface Requirements	8

3.1.	User Interfaces	8

3.1.1.	LED	8

3.1.2.	Button	8

3.2.	Hardware Interface	8

3.2.1.	Electrical interface	8

3.2.2.	SCLK / SDA	10

3.2.3.	MRST IN#	10

3.2.4.	HWRST IN#	10

3.2.5.	DISABLE	10

3.3.	Software Interface	10

3.4.	Communications Interface	10

3.4.1.	Layer 1 – The Physical Layer	10

3.4.2.	Layer 2 – The Data Layer	10

3.4.3.	Layer 3 – The Network Layer	10

3.4.3.1.	Command frame format	11

3.4.3.2.	Response frame format	12

3.4.3.3.	Data Representation and Format	13

3.4.4.	Layer4 – The Transport Layer	13

3.4.4.1.	Synchronized Communications Transactions	13

3.4.4.2.	Non-Synchronized Communications Transactions	13

3.4.4.2.1.	Read Transaction without Prior Write Transaction	13

3.4.4.2.2.	Non-Overlapped Consecutive Commands	14

3.4.4.2.3.	Overlapped Consecutive Commands	14

4.	System Features	15

4.1.	Resets and Initializations	15

4.1.1.	Cold Reset	15

4.1.1.1.	Trigger	15

4.1.1.2.	Affect on Traffic	15

4.1.1.3.	Settings Restoration	15

4.1.1.4.	Initialization Time	15

4.1.2.	Warm Reset	15

4.1.2.1.	Trigger	15

4.1.2.2.	Affect on Traffic	15

4.1.2.3.	Settings Restoration	16

4.1.2.4.	Initialization Time	16

4.1.3.	Hot Reset	16

4.1.3.1.	Trigger	16

4.1.3.2.	Affect on Traffic	16

4.1.3.3.	Settings Restoration	16

4.1.3.4.	Initialization Time	16

4.1.4.	Flow Chart	17

4.1.5.	State Diagram	17

4.1.6.	Timing Diagram	17

4.1.7.	Run-Time Functional Behaviors	17

4.2.	Setpoint Range and Factory default value	23

4.3.	Alarm Management	23

4.3.1.	Alarm events	23

4.3.2.	Alarm Threshold and Histeresys	23

4.3.3.	Alarm FIFO Retrieve	23

4.3.4.	Alarm Status Retrieve	23

4.3.5.	Flow Chart	23

4.3.6.	State Diagram	23

4.3.7.	Timing Diagram	23

4.4.	In-Field Firmware Upgrade	24

4.4.1.	General Requirements	24

4.4.2.	Firmware Upgrade Process	24

4.4.3.	Flow Chart	24

4.4.4.	State Diagram	24

4.4.5.	Timing Diagram	24

5.	Other Non-functional Requirements	25

5.1.	Safety Requirement	25

5.2.	Security Requirement	25

5.3.	Performance Requirement	25

5.4.	Business Rules	25

5.5.	User Documentation	25

6.	Other Requirements	26

Appendix A:	27

Appendix B: Analysis Models	27

Appendix C: Issues List	27

- Introduction

Identification

Purpose

This document specifies the firmware requirement of the 429F Modules for Oplink   Corporation.

Document Conventions

Intended Audience and Reading Suggestions

Project Scope

The firmware product is embedded into 429F Module performing necessary functionalities to be specified in the following sections.

Reference

Fim3_tester_system_main.pdf

- Overall Description

Overview Description

NA

Product Features

NA

User Classes and Characteristics

NA

Operating Environment

NA

Hardware Platform

Freescale Semiconductor MK20DN512ZVLL10

Operating System

NA

Other Software Components or Applications

NA

Design and Implementation Constraints

<Describe any items or issues that will limit the options available to the developers. These might include: corporate or regulatory policies; hardware limitations (timing requirements, memory requirements); interfaces to other applications; specific technologies, tools, and databases to be used; parallel operations; language requirements; communications protocols; security considerations; design conventions or programming standards (for example, if the customer’s organization will be responsible for maintaining the delivered software).>

Assumptions and Dependencies

<List any assumed factors (as opposed to known facts) that could affect the requirements stated in the SRS. These could include third-party or commercial components that you plan to use, issues around the development or operating environment, or constraints. The project could be affected if these assumptions are incorrect, are not shared, or change. Also identify any dependencies the project has on external factors, such as software components that you intend to reuse from another project, unless they are already documented elsewhere (for example, in the vision and scope document or the project plan).>

- External Interface Requirements

User Interfaces

LED

D5

Button

NA

Hardware Interface

Electrical interface

Table 0-1 Control-pin functions of external interface

| Name of function | Abbreviation | I/O | Pin number | contents | Description |
| --- | --- | --- | --- | --- | --- |
|  | ONLINE1_1 | I |  |  |  |
|  | ONLINE2_1 | I |  |  |  |
|  | ONLINE3_1 | I |  |  |  |
|  | ONLINE4_1 | I |  |  |  |
|  | MCS1_DONE_OUT | I |  | Command done | This signal is generated by the module to indicate that the command has been executed. This signal is an active high strobe of minimum 220 ns width. |
|  | MCS2_DONE_OUT | I |  | Command done | This signal is generated by the module to indicate that the command has been executed. This signal is an active high strobe of minimum 220 ns width. |
|  | PWR1 | O |  |  |  |
|  | PWR2 | O |  |  |  |
|  | DISABLE1_1 | O |  |  |  |
|  | DISABLE2_1 | O |  |  |  |
|  | DISABLE3_1 | O |  |  |  |
|  | DISABLE4_1 | O |  |  |  |
| Software/ Hardware RST input | /HW_RST | O |  | Software reset/ Hardware reset | By putting it to Low-level, it launches Software/Hardware reset. The processor is reset and the registers are initialized, but the switch status will not change. During the reset the optical traffic will not be affected.At least 2us. |
| Master RST input | /MRST_IN | O |  | Master Reset | By putting it to Low-level, it launches the Master reset. After the reset, all switches are in the shut off status, i.e. they will be maximum attenuation. When the power is on, the same reset is performed as Master Reset. |
|  | M_1X64_RESET1 | O |  |  |  |
|  | M_1X64_RESET2 | O |  |  |  |

SCLK / SDA

MRST IN#

HWRST IN#

DISABLE

Software Interface

Communications Interface

Layer 1 – The Physical Layer

Serial Communication Interface

The settings of the RS232 interface are: 8 data bits, 1 start bit, 1 stop bit, no parity bit, and no flow control. The baud rate is 115,200 bps. The 429F module use 5 serial communication interface.

3#: memsswitch

MCU:

MK20DN512ZVLL10    UART0

UART1

UART4

UART2

UART3

4#: memsswitch

PC

1#:Z4671 MCS

2#:Z4671MCS or OPM or ITLA SOURCE

Serial Peripheral Interface

SPI communication interface is conjunct with SCLK, MISO, MOSI,WP and CS pin. The serial clock is 8M.

I-square-C-bus

N/A

Layer 2 – The Data Layer

All data sent to / read from the module is encoded in US ASCII format except firmware upgrade. The data value and range of each command will be specified in corresponding sections.

Layer 3 – The Network Layer

Communications between host and 429F is carried out in the form of message frame. The message sent from host to 429F is defined as Command. The message sent from 429F back to host is defined as Response.

      - Command frame format

Every command packet has the same format:

<Command>[Parameter] [Data] <CR>

A <field> always presents in the packet, while a [field] is optional.

- Command, Parameter and Data are strings of ASCII letter(s), which is case-insensitive.

- Command terminator is either <CR>, In the case of <CR>, the command is executed on receipt of the <CR> characters. If the coming character is <LF>, it is ignored.

The Command, Parameter and Data are separated by the blank (0x20).

Serial Communication Interface

Same as above description.

Serial Peripheral Interface

On communication between the SPI master and MCU, three packets are exchanged on the bus as shown below. Incoming data on the SPI-TX line are sampled on the rising edge of the SPI_CK and serial data transmitted on the SPI_RX line are shifted out on the falling edge of SPI_CK.

Command, parameter and data payload field:

| Cmd | Description |
| --- | --- |
| 0 | Command character 0 |
| 1 | Command character 1 |
| … | … |
| n | Command character n |

| Para | Description |
| --- | --- |
| 0 | Command character 0 |

| Data | Description |
| --- | --- |
| 0 | Data character 0 |
| … | … |
| n | Data character n |

| End | Description |
| --- | --- |
| 0 | 0x0D |

I-square-C-bus

      - Response frame format

Every response packet is in the format of:

[Status][Data]<CR>

A <field> always presents in the packet, while a [field] is optional.

- If a “Set” command is executed successfully, [Status] is ‘OK’.

- If a command packet contains invalid Command, the [Status] is ‘Invalid command’.

- If a command packet contains invalid Parameter, the [Status] is ‘Invalid parameter’.

- If a command packet contains invalid Data, the [Status] is ‘Invalid data’.

- For “Get” command, the inquired information is returned in [Data] as a combination of alphanumerical letters.

- Each response should be terminated with “<CR> (0x0D)”.

- If the data are more than 1, they are separated by the character ‘,’.

- The module doesn’t echo characters sent to it.

Serial Communication Interface

Same as above description.

Serial Peripheral Interface

Response message frame:

|  | Data | Description |
| --- | --- | --- |
| Data 0 | 0 | Data character 0 |
|  | … | … |
|  | n | Data character n |
| Data 1 | 0 | Data character 0 |
|  | … | … |
|  | n | Data character n |
| … |  |  |
| Data n | 0 | Data character 0 |
|  | … | … |
|  | n | Data character n |

I-square-C-bus

      - Data Representation and Format

- All the bytes in Command and Response frame are in ASCII format unless otherwise specified.

- All data values in “Data” field are decimal digit unless otherwise specified.

- All data in other fields are in unsigned character, unless otherwise specified.

Layer4 – The Transport Layer

The serial communication is always initiated by host in the pair of command-response. The module shall never send unsolicited responses.

      - Synchronized Communications Transactions

Host writes a command to the 429F;Host gets the response from the 429F .

Then host can write another command to the 429F;

      - Non-Synchronized Communications Transactions

Host writes a command to the 429F;Host gets the response from the 429F .

Then host can write another command to the 429F;

        - Read Transaction without Prior Write Transaction

Serial Communication Interface

NA

Serial Peripheral Interface

The host gets the response of last command.

        - Non-Overlapped Consecutive Commands

If the host send two write transactions to the module, if the second command is sent after the first one finished, the 1st command’s response will be discarded.

        - Overlapped Consecutive Commands

Host send the command to the module, if the module is on busy status, discard this command.

- System Features

MK20DN512ZVLL10

UART4

SPI

Firmware data flow schematic diagram

ATF26004

ADC

1#: 1x64memssw

UART0

2#: 1x64memssw

UART1

PC

SCI

UART2

1#:Z4671MCS

UART3

开关ADG709

I/O

Power meter or

2#:Z4671MCS

or ITLA Source

USB

UART3

Resets and Initializations

Cold Reset

      - Trigger

Cold reset is triggered by Electrical power cycling

      - Affect on Traffic

During cold reset, traffic is affected.

      - Settings Restoration

      - Initialization Time

After cold resetting, the module should be ready for communications and control within 500ms.

Warm Reset

      - Trigger

NA

      - Affect on Traffic

NA

      - Settings Restoration

NA

      - Initialization Time

NA

Hot Reset

      - Trigger

Hot reset is triggered by:

a)	Command soft reset.

      - Affect on Traffic

During hot reset, traffic shall not be affected, because all electronic circuitry shall maintain its current state except the MCU firmware.

After hot reset, traffic is not affected either, because the setting of parameters should be restored to previous field setting.

      - Settings Restoration

After hot reset, the setting of all configurable parameters should be restored to previous field setting.

      - Initialization Time

After hot reset, the 429F module should be ready for communications and control within 500 ms.

Flow Chart

Fig. 4.1.4.1 Follow chart of reset

State Diagram

Timing Diagram

Run-Time Functional Behaviors

| IDX | Cmd | Para | Data0 | Data1 | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | HELP | NA | NA | NA | List all commands |
| 2 | INFO | … | … | … | Get the information of the 429F and products |
| 3 | RST | NA | NA | NA | The module will perform soft reset after response “OK” |
| 4 | FWDL | … |  |  | the Xmodem upgrade |
| 5 | MT | NA | NA | NA | Get the temperature of the board |
| 6 | ADC | NA | NA | NA | Get all ADC value |
| 7 | VOL | NA | NA | NA | Get the power’s voltage |
| 8 | PD | NA | NA | NA | Get PD monitor power |
| 9 | SFP | … | … | … | Set SFP laser or get its status |
| 10 | SW | … | … | … | Set switch or get switch status |
| 11 | ITLA | … | … | … | read ITLA configure,or set/get ITLA |
| 12 | PATH | … | … | … | Set optical path thru all specified switches position |
| 13 | OPM | … | … | … | Set/get the parameter of the optical power meter |
| 14 | SCAN | NA | NA | NA | Scan according to the Table |
| 17 | RESULT | NA | NA | NA | Read the result of scan |
| 18 | ALM | NA | NA | NA | Read status of alm |
| 19 | Trans | 1~2 | NA | NA | 对串口2或者串口3透传 |
| 20 | $$ | NA | NA | NA | 退出透传 |

- Help

List all commands.

- Info

Get the information of the 429F module and products.

Command: Info

Response: Basic information of the 429F and products

Example:

TX: Info

RX: Type: 429F

BOOT version: 0.0.0.0,

FW version: 0.0.1.4,

HW version: 1.0,

Manufacturing Date: 03/25/2015,

| Command | Object |
| --- | --- |
| info | 读本机信息 |
| info 1 | 读1#1x18 switch 信息 |
| info 2 | 读2#1x18 switch 信息 |
| info 3 | 读1#1X64 memsswitch tester或1#1x64mecswitch 版本 信息 |
| info 4 | 读2#1X64 memsswitch tester或2#1x64mecswitch 版本信息 |
| info 5 | 读OPM信息 |
| info 6 | 读ITLA 信息 |

Tx：info 4

Rx：OE0B 1x64 Switch Tester Version:2.0.0.0（only for 1X64 memsswitch ）

Tx：info 1

Rx：AAA     V200T03

......

Note:the information format of the products is same as the above.

- Rst

This command will let the 429F reset itself after send “OK” to the host.

Command: Rst

Response: Status

Example:

Perform soft reset.

TX: Rst

RX: OK

- Fwdl

Command: Fwdl (x)

Response: ‘C’

Example:

Use XMODEM  upgrade the firmware.

TX: Fwdl

RX: ‘C’

| Command | Object |
| --- | --- |
| FWDL | 升级本机FW |
| FWDL 0 | Download table of scan |
| FWDL 1 | 升级1#1x18 switch FW |
| FWDL 2 | 升级2#1x18 switch FW |
| FWDL 3 | 升级1#1X64 memsswitch FW（TBD） |
| FWDL 4 | 升级2#1X64 memsswitch FW(TBD) |
| FWDL 5 | 升级OPM FW |
| FWDL 6 | 升级ITLA FW |

注：table of scan（5Kbytes），详见附录A

- MT

Get the current temperature of board

Command: MT

Response: Data(enlarge 10)

- ADC

Get the ADC value of the 429F.

Command: ADC

Response: Data0,Data1,Data2, Data3, Data4, Data5

Description:

| ADC value | Signal in sch |
| --- | --- |
| Data0 | MO_5_1 |
| Data1 | MO_5_2 |
| Data2 | MO_3_1 |
| Data3 | MO_3_2 |
| Data4 | MO_3_3 |
| Data5 | temp |

Example:

TX: ADC

RX: 123456, 1234, 12, 1,34,45

- VOL

Get the monitored voltage .

Command:VOL

Response:data0,data1,data2,data3,data4(enlarge 100)

Description:

| Voltage(converted) | Signal in sch |
| --- | --- |
| Data0 | MO_5_1 |
| Data1 | MO_5_2 |
| Data2 | MO_3_1 |
| Data3 | MO_3_2 |
| Data4 | MO_3_3 |

Example:	TX:VOL

RX:501,499,300,302,303

- PD

Get PD monitor power.(输入光功率，由1#1x18的PD探测)

Command: PD

Response: data(enlarge 10)(unit:dBm)

Example:	TX:PD

RX:15

- SFP

Set the SFP laser or get its status.

Command: SFP x y (optional)

Description:

| Command |  | Response | Note |
| --- | --- | --- | --- |
| Format | Type |  |  |
| SFP x y | set | OK | 1、x:1~4; 2、Y:1 or 0 |
| SFP x | get | 1 or 0 | 1:on 0:off |

- SW

Set Switch or get its status.

Command: SW x y z (optional).

Description:

| Command |  | Response | Description |
| --- | --- | --- | --- |
| Format | Type |  |  |
| SW x y z | set | OK | Set switch type x inde y to position z. - x:switch type(1: 1x64 SW, 2: 1x18 SW, 3: 1x8 SW) 4:1x2SW; 2、y: Switch index ; 3、z: Switch position; 4、（1）1x64 Switch: SW 1 y z (x=1, y:1-2, z:1-64) (2) 1x18 Switch: SW 2 y z (x=2, y:1-64, z:1-18); y(1~32):1# y(33~64):2# (3) 1x8 Switch: SW 3 y z (x=3, y:1-2, z:1-8) (y=2,2#1x18中的1x8)，其它为1#1x18中的1x8； |
| SW x y | set | TX:sw 4 1 RX:OK | Only x=4; Single switch: SW 4 y(y:1-2) |
|  | get | Z position. Example: TX:sw 1 2 RX:63 TX:sw 2 63 RX:17 | Get switch type x index y channel z position. x: Switch type, 1: 1x64 SW, 2: 1x18 SW,3:1x8SW y: Switch index , 1x64 Switch: SW 1 y (x=1, y:1-2) 1x18 Switch: SW 2 y (x=2, y:1-64) 1x8 Switch: SW 3 y； Y=1:1#1x18中的1x8的位置；（也包括不带y） Y=2:2#1x18中的1x8的位置； |
| SW x | get | Y port z position Example: TX:sw 3 RX:1 4 | Get switch type x index y port z position. x: Switch type, only x=3,3: 1x8 SW |

- ITLA

Read Configure or Set/get  the parameter of ITLA.

Command: ITLA

Description:

| Command |  | Response | Note |
| --- | --- | --- | --- |
| Format | Type |  |  |
| ITLA x y | set | Example: TX: ITLA 1 15502 RX:OK | x:(Y:from PC *10),to product(/10) 1-set wavelength to y(unit: nm) 2-set output power to y(unit: dBm) |
| ITLA x | get | TX: ITLA 1 RX:1550.2 | x: 1-get wavelength (unit: nm) 2-get output power (unit: dBm) |
| ITLA | get | Example: Tx:ITLA Rx:itla dev type:… |  |

- PATH(不支持mems switch1x64)

Set optical path thru all specified switches position.

Command: PATH x1 x2 y1 z1 y2 z2

Description:

| Command | Response | Note |
| --- | --- | --- |
| PATH x1 x2 y1 z1 y2 z2 | if command is success, response monitor PD power, if not, response “Fail” Example: TX:PATH 63 63 12 13 5 18 RX:153(enlarge 10) TX:PATH 63 63 12 13 5 19 RX:Fail | x1: 1x64 SW 1# channale position. x2: 1x64 SW 2# channale position. y1/y2: 1x18 Switch 1#/2# index z1/z2: y1/y2 specified 1#/2#Switch position. 1x64 Switch position (x1/x2: 1-64); 1x18 Switch index (y1/y2: 1-64, y1!=y2); 1x18 Switch position (z1/z2: 1-18); |

- OPM

Set /get the parameter of the optical  power meter.

Description:

| Command |  | Response | Note |
| --- | --- | --- | --- |
| Format | Type |  |  |
| OPM x y z | Set | if command is success, response “OK”, if not, response “Fail” | x=1:set wavelength; y:channel;z:wavelength .(unit:nm,10000x) x=2: set sample average time to z value (Unit: uS) x=4:set mode;y:channel(0~7);z:0~1; x=5:set range;y:0~7;z:0~3; (以上可以让y=1) |
| OPM x y | Get | TX:opm 1 1 RX:13100000 | x=1:get wavelength .(unit:nm)(10000x) y:channel; x=2: get sample average time (Unit: uS)； x=3:get power(unit:dBm)(1000x)； x=4:get mode; x=5:get range; x=6:reserved (以上可以让y=1) |
|  |  | TX:opm 2 1 RX:10 |  |
|  |  | TX:opm 3 1 RX:-1500 |  |

关于y、z的定义详见

- SCAN

先下载路由表，然后按照路由表自动扫描切换开关，同时保存每条通道扫描结果（功率计和PD功率）;

路由表详见附录A;

Command:SCAN

Response:”OK”

Description：1）若系统不断电，相同的路由表只需要下载一次；

2）若无路由表，response：“FAIL”;

3）若CRC错误，response：“Invalid CRC”；

4） 若表中有数据错误，response：“Invalid data”；

5）正常时，响应结果从当前剩余欲扫描通道数递减至1，然后“OK”；

- RESULT

读取扫描结果，上报所有扫描通道的功率计和PD功率（均为LONG型数据）

Command:RESULT

Response:data1，data2，data3，……

Description:数据依次为第一条通道的功率计功率，PD功率；接着第二条通道的数据……

（PD功率由1#1x18上报，功率计上报的为输出光功率）

- ALM

读取告警 状态

Command：ALM

Response:39（WORD型的十进制数）。

Description:

| Data（16位） | 1:有告警； 0：无告警 |
| --- | --- |
| 第0位 | Com0：与3# 1x64memsswitch（或1#1x64mechanical switch）通讯状态 |
| 第1位 | Com1：与4# 1x64switch（或1#1x64mechanical switch）通讯状态 |
| 第2位 | Com2：与1# 1x18switch通讯状态 |
| 第3位 | Com3：与OPM通讯状态 |
| 第4位 | Com3：与TLS通讯状态 |
| 第5位 | Com3：与2# 1x18通讯状态 |
| 第6位 | 3V1信号，监测电压状态， |
| 第7位 | 3V2信号，监测电压状态 |
| 第8位 | 3V3信号，监测电压状态 |
| 第9位 | 5V1信号，监测电压状态 |
| 第10位 | 5V2信号，监测电压状态 |
| 第11位 | 温度状态，超过70°C告警 |

- TRANS:对串口2或者3透传

TX: Trans x（x=1：对串口2透传；x=3：对串口3透传）

注：串口2和3分别接mcs1#和mcs2#；

- $$:取消透传

TX:$$(取消透传)

Setpoint Range and Factory default value

I

Alarm Management

Alarm events

NA

Alarm Threshold and Histeresys

NA

Alarm FIFO Retrieve

NA

Alarm Status Retrieve

NA

Flow Chart

NA

State Diagram

NA

Timing Diagram

NA

In-Field Firmware Upgrade

General Requirements

NA

Firmware Upgrade Process

Flow Chart

State Diagram

NA

Timing Diagram

NA

- Other Non-functional Requirements

Performance Requirement

Safety Requirement

<Specify those requirements that are concerned with possible loss, damage, or harm that could result from the use of the product. Define any safeguards or actions that must be taken, as well as actions that must be prevented. Refer to any external policies or regulations that state safety issues that affect the product’s design or use. Define any safety certifications that must be satisfied.>

Security Requirement

<Specify any requirements regarding security or privacy issues surrounding use of the product or protection of the data used or created by the product. Define any user identity authentication requirements. Refer to any external policies or regulations containing security issues that affect the product. Define any security or privacy certifications that must be satisfied.>

Performance Requirement

<Specify any additional quality characteristics for the product that will be important to either the customers or the developers. Some to consider are: adaptability, availability, correctness, flexibility, interoperability, maintainability, portability, reliability, reusability, robustness, testability, and usability. Write these to be specific, quantitative, and verifiable when possible. At the least, clarify the relative preferences for various attributes, such as ease of use over ease of learning.>

Business Rules

<Specify any business regulations here.>

User Documentation

<List the user documentation components (such as user manuals, on-line help, and tutorials) that will be delivered along with the software. Identify any known user documentation delivery formats or standards.>

- Other Requirements

<Define any other requirements not covered elsewhere in the SRS. This might include database requirements, internationalization requirements, legal requirements, reuse objectives for the project, and so on. Add any new sections that are pertinent to the project.>

Appendix A:

Table of scan：bin文件格式

| 序号 | 表头(4Bytes) “OPLK” |  |  |  | CRC32(4Bytes) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0x4F | 0x50 | 0x4C | 0x4B | HH | HL | LH | LL |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| … |  |  |  |  |  |  |  |  |
| 576 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| … | 0xFF | 0xFF | 0xFF | 0xFF | 0xFF | 0xFF | 0xFF | 0xFF |
| 639 | 0xFF | 0xFF | 0xFF | 0xFF | 0xFF | 0xFF | 0xFF | 0xFF |

注：

- 表的大小最大5K（5120字节）；以1K xmodem方式下载；

- 表头4个字节（上图绿色部分）；最多576通道，每个通道8Bytes，共576*8=4608 Bytes(上图黄色部分)；

每个通道控制顺序为：1*8SW;1x2SW;1x64SW1;1x64SW2;in:MCS1#or2#SW;out:MCS1#or2#SW;

in:MCS1#or2#Chn;out:MCS1#or2#Chn;

- CRC32共 4个字节（上图绿色部分）；

- 多余（5120-4608-4-4=504）字节以0xFF填充（上图蓝色部分），可以只填一个0xFF;

5、CRC32的计算从序号1到序号639的数据。

Appendix B: Analysis Models

<Optionally, include any pertinent analysis models, such as data flow diagrams, class diagrams, state-transition diagrams, or entity-relationship diagrams.>

Appendix C: Issues List

< This is a dynamic list of the open requirements issues that remain to be resolved, including TBDs, pending decisions, information that is needed, conflicts awaiting resolution, and the like.>